Data Collection Worksheet

Please Note: The Data Collection Worksheet (DCW) is a tool to aid integration of a PhenX protocol into a study. The PhenX DCW is not designed to be a data collection instrument. Investigators will need to decide the best way to collect data for the PhenX protocol in their study. Variables captured in the DCW, along with variable names and unique PhenX variable identifiers, are included in the PhenX Data Dictionary (DD) files. Sequence for task functional magnetic resonance imaging

(Task fMRI)

TA: 6:1

PAT: Off

Voxel size: 3.0 x 3.0 x 3.0 mm

Rel. SNR: 1.00

Properties

Prio Recon
Before measurement
After measurement.......................................OffLoad to viewer.......................................OnInline movie.......................................OffAuto store images.......................................OnLoad to stamp segments.......................................OnLoad images to graphic segments.......................................OffAuto open inline display.......................................OnStart measurement without further preparation.......................................OffWait for user to start.......................................OnStart measurements.......................................single

Routine

Slice group 1Slices.......................................47Dist. Factor.......................................0%Position.......................................R3.0 A3.0 H0.0Orientation.......................................T > C-12.5Phase enc. dir........................................A >> PRotation.......................................0.00 degPhase oversampling.......................................0%FoV read.......................................216 mmFoV phase.......................................100.0%Slice thickness.......................................3.00 mmTR.......................................3000 msTE.......................................30 msAverages.......................................1Concatenations.......................................1Filter.......................................Prescan NormalizeCoil elements.......................................HEA;HEP

Contrast

MTC.......................................OffFlip angle.......................................85 degFat suppr........................................Fat suppr.Averaging mode.......................................Long termReconstruction.......................................MagnitudeMeasurements.......................................124Multiple series.......................................Each measurement

Resolution

Base resolution.......................................72Phase resolution.......................................100%Phase partial Fourier.......................................OffInterpolation.......................................Off

PAT mode.......................................NoneMatrix Coil Mode.......................................Auto (CP)Distortion Corr........................................OffUnfiltered images.......................................OffPrescan Normalize.......................................OnRaw filter.......................................OnElliptical filter.......................................OffHamming.......................................Off

Geometry

Multi-slice mode.......................................InterleavedSeries.......................................InterleavedSpecial sat........................................None

System

Body .......................................OffHEP .......................................OnHEA .......................................On

Positioning mode .......................................REFTable position .......................................HTable position .......................................0 mmMSMA .......................................S - C - TSagittal .......................................R >> LCoronal .......................................A >> PTransversal .......................................F >> HCoil Combine Mode .......................................Sum of SquaresAutoAlign .......................................Head > Brain AtlasAuto Coil Select .......................................Off

Shim mode .......................................StandardAdjust with body coil .......................................OffConfirm freq. adjustment .......................................OffAssume Silicone .......................................Off? Ref. amplitude 1H .......................................0.000 VAdjustment Tolerance .......................................AutoAdjust volumePosition .......................................R3.0 A3.0 H0.0Orientation .......................................T > C-12.5Rotation .......................................0.00 degR >> L .......................................216 mmA >> P .......................................216 mmF >> H .......................................141 mm

Physio

1st Signal/Mode.......................................None

BOLD

GLM Statistics.......................................OffDynamic t-maps.......................................OffStarting ignore meas.......................................0Ignore after transition.......................................0Model transition states.......................................OffTemp. highpass filter.......................................OffThreshold .......................................4.00Paradigm size.......................................3Meas[1].......................................BaselineMeas[2].......................................BaselineMeas[3].......................................ActiveMotion correction.......................................OffSpatial filter.......................................Off

Sequence

Introduction.......................................OffBandwidth .......................................2240 Hz/PxFree echo spacing.......................................OffEcho spacing.......................................0.51 msEPI factor.......................................72RF pulse type.......................................NormalGradient mode.......................................FastDummy Scans.......................................0FFT Scale Factor.......................................1.00

Protocol source: https://www.phenxtoolkit.org/protocols/view/662601